APD Set Up, Going on & Coming off

Procedure for Patients and Staff

# Set up Procedure

- Put used drain line down toilet.  Ensure clamp is open.

- Give hands an ordinary wash.

- Gather equipment ie Cassette, Fluid (check expiry, date, volume and strength), Handrub, Connection shields x 2, Minicaps x 3, Opticaps x 2, tape, Drainage lines & Surface wipe x 1.

- Switch off machine and unplug from mains.

- Damp dust machine with soap and water then clean surface with Surface wipe.

- Plug machine in and switch on. Press ‘Go’ to start.  The display will read ‘Load the set’.  Open door.

- Wash hands then apply Handrub.

- Remove Fluid from outer wrappers and place one bag on top of machine and the others to the side.

- Check for clarity and leaks.

- Open Cassette packet and new Drain line, then apply Handrub.

- Close all clamps (6)

- Load Cassette, close door and hang Organiser on to door hook.

- Press Go.  The display will read ‘self testing’ and when complete will change to ‘connect bags’ flashing.

- Attach Drain line to first line on right of Organiser.

- Apply Handrub.

- Remove tag from heater bag and connect to red clamped line. Break green seal.  Open clamp.

- Remove tag(s) from supply bag(s) and connect to white clamped line(s).  Break green seal(s).  Open clamp(s).

- If using last bag fill / Extaneal remove tag from bag and connect to blue clamped line. Break green seal.  Open clamp.

- Open clamp on machine patient line.

- Connect new and used drain lines.

- Press Go.  The display will read ‘Priming’ and when complete will change to ‘connect yourself/ check patient line’.

- Close machine patient line clamp.

# Going on

- Take out Miniset from clothing.

- Wash hands and apply Handrub

- Open Connection Shield  Packet and apply Handrub.

- Remove blue tag from machine patient line.

- Remove Minicap from Miniset.

- Connect machine patient line to Miniset.

- Apply Connection shield and twist 5 or 6 times.

- Open Twist clamp on Miniset and machine patient line clamp.

- Press ‘Go’.  The display will read ‘Initial Drain’.

# Coming Off Optichoice

- Following drain, then fill,  the machine will read

“day dwell 1”.

- Close Miniset and clamp machine patient line.

- Open Opticap Packet & apply Handrub.

- Remove connection shield.

- Unscrew machine patient line, place in Organiser.

- Pick up Opticap and apply Minicap end to Miniset.

- Flip blue cover off Opticap and screw Opticap onto machine patient line.

- Replace in Organiser.

# Going On NIGHT TIME

- Take out Miniset from clothing.

- Wash hands & apply Handrub.

- Open Connection shield packet & apply Handrub.

- Remove Opticap from machine patient line.

- Remove Minicap from Miniset.

- Connect machine patient line to Miniset.

- Apply Connection shield and twist 5 or 6 times.

- Open Twist clamp on miniset and machine patient line clamp.

## Scroll down to “press go to continue”

- Press go twice machine will read “day drain 1 of 1”.

- Machine will now continue with night time therapy.

# Coming OFF  at end of therapy

- At the end of treatment, the display will read ‘End of Therapy’.

- Scroll down and record:	Initial drain

### Total UF

# Average dwell

- Scroll up until the display reads ‘End of Therapy’.

- Press ‘Go’.  The display will read ‘Close all clamps’.

- Close Twist Clamp on Miniset, then close all other clamps.

- Press ‘Go’.  The display will read ‘Disconnect yourself’.

- Apply Handrub liberally.

- Open new Minicap packet.

- Apply Handrub.

- Disconnect machine patient line and drop, keep hold of Miniset and apply new Minicap.

- Press ‘Go’.  Machine will read ‘turn me off’.

- Open door and remove cassette.

- Switch off machine.

Amended 31/12/99 Amended 18/12/01 G.Owen

Amended & full revision and filename change 21/1/03 G Owen.

Amended by Gill Owen June 06. Amended Gill Owen June 2007. Revised G Owen 2016